parallel Computing 2025

M.Sc. Embedded Systems

Report Part 2

Nov 2025

Group Members:

| Mehdi Moallemkolaei | 153010947 | mehdi.moallemkolaei@tuni.fi |
| --- | --- | --- |
| Md Ashfak Haider Nehal | 153045077 | mdashfakhaider.nehal@tuni.fi |

Contents

7. Part 2	3

7.2. Porting the project to OpenCL	3

7.2.1. Benchmarking:	4

7.2.2. WG Sizes:	5

7.2.3. Analyzing:	6

7.2.4. Window Sizes - OpenCL vs. C (CPU)	7

7.2.5. Bonus - Optimization	8

7.2.5.1. Bonus in Bonus	8

8. Feedback	9

8.1. Pros	9

8.2. Improvable	9

8.3. What we learned	9

8.4. Challenges	9

8.5. Work Hours	9

9. AI usage declarations	9

# 7. Part 2

## 7.2. Porting the project to OpenCL

Summary of what we did:

- Enabled OpenCL in the CMake file.

- Created a separate kernel file parallel.cl that implements the graphics routine on the GPU based on the same logic we had earlier in Part 1. Like:

  - Pixel Loop

  - Black-hole Test with no sqrt

  - Mouse Position

  - …

- Improved only parallelGraphicsEngine() on the source code with OpenCL calls. Upload satellite positions, set kernel args, launch NDRange, read pixels back.

- OpenCL init in init() and cleanup in destroy(), with explicit error checks after every OpenCL API call via CL_CHECK.

- Physics remains OpenMP on CPU, and only graphics is moved to OpenCL.

Codes related to OpenCL:

- CMakeLists.txt

- find_package(OpenCL REQUIRED)

- Link ${OpenCL_LIBRARIES} and copy parallel.cl after build.

- parallel.c

- init()

- parallelGraphicsEngine()

- destroy()

- parallel.cl

- Kernel shade(...) computes per-pixel color

How to change the work-group size (WG) in our code:

We kept WG size editable in the source file, here:

static size_t WGX = 16;

static size_t WGY = 16;

So, we can change these values to test each Work-Group size.

Build & run (used for all measurements):

- Windows / Visual Studio (Release, x64)

  - CMake config: OpenMP + OpenCL + SDL2 (as in CMakeLists.txt)

  - Build: X64-Release configuration

  - The executable prints:

    - chosen platform/device (like in the Lab: NVIDIA CUDA / GeForce RTX 4070)

    - preferred WG multiple and max WG size (like 32 and 256)

### 7.2.1. Benchmarking:

Include these results in the same TABLE that you used in Part 1:

1. The Original C Version on CPU without optimizations?

2. The Original C version on CPU with best optimizations?

3. The OpenCL version on GPU, WG size 1x1?

4. The OpenCL version on GPU, WG size 4x4?

5. The OpenCL version on GPU, WG size 8x4?

6. The OpenCL version on GPU, WG size 8x8?

7. The OpenCL version on GPU, WG size 16x16?

| NO. | MODE | Benchmark |  |  |
| --- | --- | --- | --- | --- |
|  |  | Physics | Graphics | Total |
| Lab Setup |  |  |  |  |
| 1 | CPU – No Optimizations - Debug | 164 ms | 1251 ms | 1417 ms |
| 2 | CPU – No Optimizations - Release | 117 ms | 557 ms | 676 ms |
| 3 | CPU – Best Optimizations | 5 ms | 16 ms | 23 ms |
| OpenCL |  |  |  |  |
| 1 | WG size 1x1 | 5 ms | 13 ms | 21 ms |
| 2 | WG size 4x4 | 5 ms | 2 ms | 9 ms |
| 3 | WG size 8x4 | 5 ms | 1 ms | 8 ms |
| 4 | WG size 8x8 | 5 ms | 1 ms | 8 ms |
| 5 | WG size 16x16 | 5 ms | 1 ms | 8 ms |
| 6 | WG size 32x32 | 5 ms | 1 ms | 7 ms |
| Our Setup |  |  |  |  |
| 1 | CPU – No Optimizations - Debug | 188 ms | 1369 ms | 1561 ms |
| 2 | CPU – No Optimizations - Release | 130 ms | 617 ms | 750 ms |
| 3 | CPU – Best Optimizations | 8 ms | 24 ms | 34 ms |
| OpenCL |  |  |  |  |
| 1 | WG size 1x1 | 7 ms | 30 ms | 40 ms |
| 2 | WG size 4x4 | 7 ms | 3 ms | 12 ms |
| 3 | WG size 8x4 | 7 ms | 2 ms | 11 ms |
| 4 | WG size 8x8 | 7 ms | 2 ms | 11 ms |
| 5 | WG size 16x16 | 7 ms | 2 ms | 11 ms |
| 6 | WG size 32x32 | 7 ms | 2 ms | 11 ms |

* These benchmarks are only from the first 10 to 20 frames. While later frames show an increase in the delays. (default dimension: 1920x1024)

### 7.2.2. WG Sizes:

Make your code so that you can change the WG size easily afterwards because you might be asked to demonstrate different sizes when showing the code to the assistant.

To make the work-group (WG) size adjustable for demonstrations, we defined the WG parameters as static variables in the source code:

static size_t WGX = …; static size_t WGY = …;

This allows us to quickly change WG configurations during testing or demonstration without recompiling major parts of the code.

#### 7.2.2.1 Bonus:

If some work group size or sizes did not work, try to explain why they did not work.

During our lab tests, all WG configurations up to 16x16 worked correctly, which we initially considered a positive sign. We then extended testing to 32x32, which also executed successfully and showed similar performance results to 16x16. However, increasing the WG dimensions further (32x33 or 64x64) caused program crashes with the following error: OpenCL error -54 at…

This corresponds to CL_INVALID_WORK_GROUP_SIZE (OpenCL error -54), indicating that the requested local size exceeded the maximum supported by the device or kernel. (We will tell which one!)

BUT there is a confusing point we faced. The “CL_KERNEL_WORK_GROUP_SIZE” in the source code is supposed to show the max WG size, and it shows 256:

OpenCL platform: NVIDIA CUDA ¦ vendor: NVIDIA Corporation OpenCL device : NVIDIA GeForce RTX 4070 ¦ type: GPU Preferred WG multiple: 32 ¦ Max WG size: 256

It seems normal, but the problem is that we tried 32x32 and the executable file still works!
32x32 is more than 256, which is mentioned as the max WG size.

It was confusing! ‍

To verify this, we searched a lot and we guess this is the only reason we could provide. We know that Device-wide max and Kernel-specific max are two different concepts.

Currently, “CL_KERNEL_WORK_GROUP_SIZE” shows the Kernel-specific max equal to 256. But we added the third factor, “CL_DEVICE_MAX_WORK_GROUP_SIZE”, to show what the real GPU max WG is. Here is the result:

OpenCL platform: NVIDIA CUDA ¦ vendor: NVIDIA Corporation OpenCL device : NVIDIA GeForce RTX 4070 ¦ type: GPU Preferred WG multiple: 32 ¦ Kernel Max WG size: 256 ¦ Device max WG size: 1024

So, as we can see, the real max WG size supported by the device is 1024, which is also supported by “Preferred WG multiple: 32” (32x32=1024).

### 7.2.3. Analyzing:

Try to analyze the performance with different work group sizes. What might explain the differences?

To better understand how work-group (WG) size affects execution time, we tested several configurations while keeping all other parameters constant. These are the tested values:

| NO. | WGX x WGY | Total | works? | Observation |
| --- | --- | --- | --- | --- |
| 1 | 1x1 | 1 | Yes | Basic performance and GPU utilization |
| 2 | 8x8 | 64 | Yes | Stable execution, moderate GPU utilization |
| 3 | 16x16 | 256 | Yes | Better performance due to improved parallel occupancy |
| 4 | 32x32 | 1024 | Yes | Best observed performance, highest GPU occupancy |
| 5 | 32x33 | >1024 | No | Exceeds device limit |
| 6 | 64x64 | >1024 | No | Exceeds device limit |

Performance improves as the work-group size increases up to the hardware limit.

The observed differences arise mainly from:

- Thread-level Parallelism
Larger WGs allow more concurrent threads to run, hiding memory latency and better utilizing GPU cores.

- Warp Alignment:
Since NVIDIA warps are 32 threads wide, WG sizes that are multiples of 32 (32x16, 32x32) achieve perfect warp alignment and avoid idle lanes.

- Memory Access Patterns:
Neighboring threads in a WG often access adjacent pixels, leading to better memory coalescing in larger, well-aligned groups.

- Driver/Runtime Scheduling Overhead:
Too many small groups increase kernel launch and synchronization overhead, lowering performance.

- Hardware Limits:
When WGX x WGY > 1024, the GPU rejects the launch due to exceeding the maximum threads per work-group, triggering “CL_INVALID_WORK_GROUP_SIZE”.

Performance scaled positively with WG size up to 32x32, after which it crashed.

This matches NVIDIA’s architectural preference for warp-aligned groups and confirms that 1024 threads per workgroup is the practical and theoretical optimum for this kernel on our RTX 4070 GPU.

### 7.2.4. Window Sizes - OpenCL vs. C (CPU)

Try reducing the window size to a minimal size, such as 80x80. The WINDOW WIDTH and WINDOW HEIGHT are defined at the beginning of the code that controls this.

Fastest WG size for OpenCL version is 16x16 while 32x32 shows the same performance. So, we can continue with 32x32:

| NO. | MODE | Benchmark |  |  |
| --- | --- | --- | --- | --- |
|  | Window Size | Physics | Graphics | Total |
| Lab Setup |  |  |  |  |
| OpenMP - CPU |  |  |  |  |
| 1 | 1920 x 1024 | 5 ms | 16 ms | 22 ms |
| 2 | 1440 x 720 | 5 ms | 8 ms | 14 ms |
| 3 | 720 x 384 | 5 ms | 2 ms | 7 ms |
| 4 | 80 x 80 | 5 ms | 0 ms | 5 ms |
| 5 | 20 x 20 | 5 ms | 0 ms | 5 ms |
| 6 | 5 x 5 | 5 ms | 0 ms | 5 ms |
| OpenCL with WG Size 32x32 |  |  |  |  |
| 1 | 1920 x 1024 | 5 ms | 1 ms | 8 ms |
| 2 | 1440 x 720 | 5 ms | 0 ms | 6 ms |
| 3 | 720 x 384 | 5 ms | 0 ms | 5 ms |
| 4 | 80 x 80 | 5 ms | 0 ms | 5 ms |
| 5 | 20 x 20 | 5 ms | 0 ms | 5 ms |
| 6 | 5 x 5 | 5 ms | 0 ms | 5 ms |
| Our Setup |  |  |  |  |
| OpenMP - CPU |  |  |  |  |
| 1 | 1920 x 1024 | 8 ms | 24 ms | 35 ms |
| 2 | 1440 x 720 | 8 ms | 12 ms | 21 ms |
| 3 | 720 x 384 | 8 ms | 3 ms | 12 ms |
| 4 | 80 x 80 | 8 ms | 0 ms | 8 ms |
| 5 | 20 x 20 | 8 ms | 0 ms | 8 ms |
| 6 | 5 x 5 | 8 ms | 0 ms | 8 ms |
| OpenCL with WG Size 32x32 |  |  |  |  |
| 1 | 1920 x 1024 | 8 ms | 2 ms | 11 ms |
| 2 | 1440 x 720 | 8 ms | 1 ms | 9 ms |
| 3 | 720 x 384 | 8 ms | 0 ms | 8 ms |
| 4 | 80 x 80 | 8 ms | 0 ms | 8 ms |
| 5 | 20 x 20 | 8 ms | 0 ms | 8 ms |
| 6 | 5 x 5 | 8 ms | 0 ms | 8 ms |

If the OpenCL version running on the GPU was slower than C on CPU with this image size, what is the reason for this?

In our measurements, total frame time is equal to = physics (CPU/OpenMP) + graphics (GPU/OpenCL)

For tiny images (5x5, 20x20, 80x80), the OpenCL graphics kernel is essentially negligible (~0 ms), but the physics step remains the same “ms” because it performs PHYSICSUPDATESPERFRAME Euler iterations on the CPU.

We left physics on the CPU; therefore, for small images, the CPU physics dominates, and GPU advantages appear only when the image is large enough to show its power.

So, OpenCL on the GPU isn’t slow here. It is mostly hidden under the constant CPU physics cost at small resolutions. As resolution grows, GPU parallelism reduces the graphics part substantially, and the total frame time becomes better than the CPU-only version.

### 7.2.5. Bonus - Optimization

As a final (and the most significant) bonus question, you are now free to implement any optimizations to your program. How fast can you get it to run?

We worked diligently to achieve optimal performance, and all the benchmarks above related to OpenCL are derived from the optimized version of the code.

What we have done so far:

Part2 modifications:

- Enabled OpenCL in CMake file and use in the source code, plus implementing NVIDIA GPU selection

  - Verified GPU selection and capability by printing platform, device name, and vendor information.

- Created a practical OpenCL kernel (parallel.cl):

  - Ported the heavy pixel-coloring computation from the CPU’s parallelGraphicsEngine() to an OpenCL kernel named “shade”.

  - Ensured correctness by comparing against the sequential reference

- Optimized kernel and data flow:

  - Enabled build flags -cl-fast-relaxed-math, -cl-mad-enable, and -cl-unsafe-math-optimizations to let the compiler fuse operations and reduce floating-point latency.

  - Updated only position buffers each frame, while keeping satellite color data constant on the GPU

  - Used persistent OpenCL buffers to minimize memory allocations and transfers across frames.

- Made work-group size fully configurable:

  - Introduced global variables WGX and WGY to allow dynamic adjustment of work-group dimensions without recompilation.

  - Added queries for both CL_KERNEL_WORK_GROUP_SIZE and CL_DEVICE_MAX_WORK_GROUP_SIZE to understand kernel-specific and device-wide limits, enabling systematic testing.

  - Validated performance and stability for multiple configurations (8×8, 16×16, 32×32) and documented that 32×32 = 1024 threads per group gave the best performance.

- Optimized overall execution pipeline:

  - Overlapped CPU and GPU workloads where possible and ensured efficient synchronization using clFinish() only when needed.

Through careful GPU selection, kernel file, memory restructuring (AoS to SoA), and configurable work-group sizing, we transformed the original CPU-bound implementation into a GPU-accelerated hybrid (CPU + GPU) system capable of high-resolution real-time simulation and rendering.

All the reported benchmarks and analyses were obtained from this optimized final version.

### 7.2.5.1. Bonus in Bonus

Add some more thoughts on how you would attempt to further improve the total performance of this system?

No idea how much these approaches would help the total performance but we can consider these:

1. Move physics to GPU:
GPU physics kernel updates positions and velocity in device memory, then SHADE kernel uses them in-place, which avoids position uploads (between cl and c file) per frame.

2. other ways of using sqrt:
In our implementation we tried to avoid sqrt, but if we are forced to use this in the mathematics, then it’s better to use fsqrt or rsqrt.

The rest ideas came to our mind were used in our solution.

# 8. Feedback

## 8.1. Pros

The exercise was well-structured and provided a clear, step-by-step path from OpenMP to OpenCL, allowing us to understand the transition from CPU-based parallelization to GPU acceleration. It was also practical that we could see measurable performance improvements and directly visualize the impact of parallel computing.

## 8.2. Improvable

Clarifying the difference between kernel-specific and device-wide work-group limits in the instructions would help avoid confusion.

## 8.3. What we learned

Learning how to port an existing OpenMP codebase to OpenCL and run it efficiently on a GPU was the most valuable part.

## 8.4. Challenges

We can not mention any specific part as the most challenging part since regarding our background, all sections of the whole assignment were kind of challenging and we tried to solve them step by step.

## 8.5. Work Hours

We two approximately spent over 60 hrs totaly on the whole assignment (Part 2: +30 hrs).

# 9. AI usage declarations

ChatGPT has been used in this course assignment only to do research on different topics, clearing up ambiguities and confusions, and learn new concepts.